|  |  | AMOFSG/7-SN No. 15 22/5/08. |
| --- | --- | --- |

AERODROME METEOROLOGICAL OBSERVATION AND FORECAST STUDY GROUP (AMOFSG)

SEVENTH MEETING

Montréal, 9 to 12 September 2008

| Agenda Item | 6: | Aerodrome forecasts |
| --- | --- | --- |

Cancellation of TAFs prior to closure of aerodrome

(Presented by Colin Hord)

SUMMARY This study note highlights the potential inefficiency in the application of ICAO SARPs in relation to the non-operational hours of aerodromes, and offers a United Kingdom perspective.

- INTRODUCTION

  - Amendment 74 to Annex 3 — Meteorological Service for International Air Navigation , effective 5th November 2008 is likely to lend some inefficiency to the cancelling of TAFs that are not  kept under continuous review, against the requirement for a minimum TAF validity period.

  - One the one hand, para 6.2.4. and 6.2.5. of Amendment 74 notes the Standard to keep forecasts under continuous review and to cancel them when this continuous review cannot be maintained. The provision of TAFs is generally reliant on the production of METARs, which itself is generally based on the operational hours of that aerodrome.

6.2.4 Meteorological offices preparing TAF shall keep the forecasts under continuous review and, when necessary, shall issue amendments promptly. The length of the forecast messages and the number of changes indicated in the forecast shall be kept to a minimum.

    - TAF that cannot be kept under continuous review shall be cancelled.

  - On the other hand however, Amendment 74 (valid from November 2008), recommends that a TAF validity period should not be less than 6 hours in duration.

Amendment 74 (part 2, effective 5th November 2008)

6.2.6 Recommendation.— The period of validity of a routine TAF should be not less than 6 hours nor more than 30 hours; this period of validity should be determined by regional air navigation agreement. Routine TAF valid for less than 12 hours should be issued every 3 hours and those valid for 12 to 30 hours should be issued every 6 hours.

  - It can be demonstrated that in some situations, these two regulations lead to inefficiency in forecasting both in terms of the forecast period and the need to cancel TAFs. An example will be given, with an explanation of how the United Kingdom overcomes such situations, along with the rationale for its decision.

- DISCUSSION

  - Example 1: TAF schedule based on the application of a minimum 6 hour TAF validity period at an non-H24 airport.

EGXX:	operational hours 0600-2300 UTC

TAF schedule: 9 hour TAFs, issued every 3 hours

Min TAF length 6 hours (as per 6.2.6., November 2008)

|  | Hour commencing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAF | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 01 | 02 | 03 | 04 | 05 |
| 0615 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0918 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1221 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1524 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1803 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0009 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0312 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TAF not issued | TAF issued & validity period | TAF available but not utilised |
| --- | --- | --- |

  - A total of 46 hours are forecast for this airport each day, but 6 of these would not be used about (13 per cent of all hours forecast). The situation in this example leads to wasted resources in forecasting for periods that  forecasters know will not be required. It also necessitates each affected TAF to then be cancelled since it cannot be kept under continuous review when the airport closes and METARs are not produced.

  - Given the significant number of aerodromes that TAF are issued for in the United Kingdom (54), this is an extremely resource intensive issue. The inefficient nature of this regulation in its current state has necessitated the United Kingdom to file a difference with ICAO to reduce the minimum TAF length to just 2 hours. This enables the validity periods of TAFs to be limited to operational hours of airports, so that they may be kept under continuous review (as per Annex 3, para 6.2.4.). As well as not forecasting for unnecessary periods, it also removes the need to cancel these TAFs, since they expire at the time the airport closes.

- For example:

- The amendment of Annex 3, para 6.2.6, from November 2008 reducing the minimum TAF validity period from 9 hours to 6 hours, goes some way to addressing this issue, but still creates periods in a TAFs validity that are not required operationally, as shown in the table above.

  - Example 2: TAF schedule, removing the limitations of applying a 6 hour minimum TAF validity period (Annex 3, par 6.2.6) at a non-H24 airport.

EGXX:	operational hours 0600-2300 UTC

TAF schedule: 9 hour TAFs, issued every 3 hours

Min TAF length: according to ability to keep TAF under continuous review (as per 6.2.4 & 6.2.5) (min 2 hours in UK).

|  | Hour commencing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TAF | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 01 | 02 | 03 | 04 | 05 |
| 0615 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0918 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1221 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1524 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1803 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 2106 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0009 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0312 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| TAF not issued | TAF issued & validity period | TAF available but not utilised |
| --- | --- | --- |

  - As can be seen, there is no need to cancel any TAFs, since their period of validity only covers the operational hours of the aerodrome. Such an action reduces the number of messages generated by a forecast office (in this case by 2 TAF CANX messages per day for this aerodrome). Similarly, there are no periods where time and effort is invested by the forecaster into parts of the TAF that are not going to be routinely required.

  - When viewed in isolation the difference in effort and efficiency may appear trivial. However, consider that in the United Kingdom, there are 55 METAR reporting aerodromes for which TAFs are produced. The daily effort preserved as a whole is significant.

  - In the United Kingdom there are 34 aerodromes in receipt of daily TAFs that would otherwise require their TAFs cancelling since they are not H24. This equates to 68 cancellation messages that would otherwise need to be issued in the United Kingdom per day, based on 2 TAF CANX messages per aerodrome, Further, there are an estimated 200 TAF forecast hours saved in the United Kingdom by removing the minimum TAF period, per day.

- ACTION BY THE GROUP

  - The group is invited to: consider changing the recommended practice in paragraph 6.2.6 of Annex 3 to commence "the period of validity of a routine TAF should be not less than 2 hours nor more than 30 hours.

- — END —